[image: image1.jpg]





                                                                                                                              Technical Bulletin

To: Per Distribution List






From: Brian Midgetté










Date: 05/16/07










Subject: Festo High Speed Solenoid
Bulletin 051607-02
ITW Dynatec recently introduced a Festo solenoid valve for use in high speed applications. The Festo valve has several advantages. It requires less power, less air consumption and is smaller, faster and quieter than the Mac 400 series solenoids currently being used on ITW Dynatec equipment. The valve is rated for 24vdc with a wattage rating of 6.5 watts. The 2.5 meter cable with connector is equipped with a LED indicator.  Over-excitation of the output voltage must not be used otherwise damage will occur. The controller must supply a true 24vdc signal.

 The valve is recommended for single module activation.       

Use of this solenoid is limited in that it is not compatible for use with the TPC-2 and DPC-2 pattern controllers. The Festo valve has electronic control circuitry that activates the TPC-2 and DPC-2 output protection circuit preventing the controller from providing an output.
Below is a comparison of the Festo and 400 series solenoid valves.

[image: image2.png]ADHESIVE W APPLICATION SOLUTIONS
IE) ECGER










